HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/01/2026 | Date: 29/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GOSP- 06 | Supervisor not using mandatory PPE (Helmet & safety glasses) while supervising grinding work. | The supervisor was instructed to always wear the mandatory helmet and safety glasses in the work area. | WPR/Foreman | 29/08/2026 | Major | Closed |
| 2 | GOSP- 06 | Hot work (cutting/grinding) area was not barricaded or protected with a fire blanket. | Instructed the Fire Watcher to properly cover the activity with a fire blanket and barricade the area. | Site Supervisor | 29/08/2026 | Major | Closed |
| 3 | GOSP- 06 | Helper was standing on an unsecured small metal piece while a fitter was grinding it, placing his foot in the line of fire. | Instructed the Fitter and Helper to secure the workpiece with a proper tool (vise/clamp) and to avoid being in the line of fire. | Electrician | 29/08/2026 | Major | Closed |

| _____________________________ Observed By: Safety officer Name: SHEMEER SHAMSUDE Date: 29/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: ALAA ABDELGHANY Date: 29/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/02/2026 | Date: 29/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | GOSP - 05 | Protruding nails were not removed or bent from dismantled wooden boxes before disposal. | Workers were instructed to remove or bend all protruding nails from wood waste before disposal. | Site Engineer | 29/08/2026 | Minor | Closed |
| 5 | GOSP - 05 | A portable water tank was found unattended at the workplace. | The unattended water tank was removed from the workplace. | Mechanical Engineer | 29/08/2026 | Minor | Closed |
| 6 | GOSP - 05 | During chipping activity, the designated Fire Watcher was absent from his post. | The Fire Watcher was instructed to remain at the hot work location and continuously monitor the activity. | Civil Engineer | 29/08/2026 | Major | Closed |

| _____________________________ Observed By: Safety officer Name: MOHAMED AARIF Date: 29/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: AL MOATASEMBELLAH Date: 29/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/03/2026 | Date: 29/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | GOSP - 05 | Scaffold tag was found with an incorrect inspection date. | The scaffold tag was corrected with the proper date as per the two-week standard. | Electrician | 29/08/2026 | Minor | Closed |
| 8 | GOSP - 05 | An air blower was being used for dust cleaning without any dust control measures (e.g., water suppression). | Effective dust control measures were implemented before work was allowed to resume. | Civil Supervisor | 29/08/2026 | Minor | Closed |
| 9 | GOSP - 05 | Personnel were off-loading steel materials at night without wearing clear safety glasses. | All personnel were instructed to wear clear safety glasses for night work. | Site Engineer | 29/08/2026 | Minor | Closed |

| _____________________________ Observed By: Safety officer Name: MOHAMED AARIF Date: 29/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: AL MOATASEMBELLAH Date: 29/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/04/2026 | Date: 29/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | GOSP - 02 | Compactor operator was working at a high speed despite hot weather conditions, increasing heat stress risk. | The operator was coached on the dangers of heat stress and instructed to work at a safe pace according to conditions. | Mechanical Engineer | 29/08/2026 | Major | Closed |
| 11 | GOSP - 02 | A compressor was in use without being properly grounded. | The compressor was immediately taken out of service until it was properly grounded. | Civil Engineer | 29/08/2026 | Major | Closed |
| 12 | GOSP - 02 | A compressor was found with a missing pre-use inspection checklist. | The supervisor was instructed not to use the equipment until a proper pre-use inspection is completed and documented. | Rigger 3 | 29/08/2026 | Minor | Closed |

| _____________________________ Observed By: Safety officer Name: RAKIB AJAMIN Date: 29/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: OSAMA ABDELRAHMAN Date: 29/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/05/2026 | Date: 29/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | GOSP - 02 | SIF Modification activity was ongoing without the required supervision. | The job was stopped until the responsible supervisor was present at the worksite. | Site Supervisor | 29/08/2026 | Major | Closed |
| 14 | GOSP - 02 | Personnel were working without an umbrella during an 'Orange Flag' (high heat) condition. | Work was stopped and umbrellas were provided to the workers as per heat stress plan requirements. | Electrician | 29/08/2026 | Major | Closed |
| 15 | GOSP - 02 | Unauthorized personnel were observed standing under a suspended load during a lifting operation. | The lifting area was immediately cleared and re-barricaded to prevent unauthorized entry. | Civil Supervisor | 29/08/2026 | Major | Closed |

| _____________________________ Observed By: Safety officer Name: RAKIB AJAMIN Date: 29/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: OSAMA ABDELRAHMAN Date: 29/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/06/2026 | Date: 30/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | GOSP - 03 | Rigging softeners used for sharp edge protection on steel structures were not long enough to protect the web slings from damage. | The rigger was instructed to use softeners of the proper length that cover all sharp edges before lifting. | Site Engineer | 30/08/2026 | Major | Closed |
| 17 | GOSP - 03 | An electrical cable was routed on the ground where a steel assembly was in progress, creating a damage risk. | The electrical cable was relocated away from the work area to protect it from damage. | Mechanical Engineer | 30/08/2026 | Minor | Closed |
| 18 | GOSP - 03 | Excavation was proceeding with a blunt pickaxe, which is an inefficient and unsafe tool for the task. | The blunt hand tool was replaced with a new, sharp pickaxe suitable for the work. | Civil Engineer | 30/08/2026 | Minor | Closed |

| _____________________________ Observed By: Safety officer Name: BILAL SAEED MUHAMM Date: 30/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: AAFAQ AHMAD Date: 30/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/07/2026 | Date: 30/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | GOSP - 03 | High Heat Stress Index (Category IV) was observed, but work continued. | The job was immediately stopped until the heat stress category returned to a safe level. | WPR/Foreman | 30/08/2026 | Major | Closed |
| 20 | GOSP - 03 | Backfill material was unloaded less than 0.6 meters from the edge of an excavation. | The operator was instructed to always maintain a minimum distance of 0.6 meters from the excavation edge when placing spoil or materials. | Site Supervisor | 30/08/2026 | Major | Closed |
| 21 | GOSP - 03 | The location mentioned on a cable locating sketch for a cutting permit was incorrect. | The permit was rescinded. The excavation point was re-scanned, and a new, correct sketch was made before work was re-authorized. | Electrician | 30/08/2026 | Major | Closed |

| _____________________________ Observed By: Safety officer Name: BILAL SAEED MUHAMM Date: 30/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: AAFAQ AHMAD Date: 30/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019

HEISCO

HSE OBSERVATION REGISTER

| Ref No: 433001/HSE-OR/08/2026 | Date: 30/08/2026 |
| --- | --- |
| CC Name: DEBOTTLENECK PRODUCTION FACILITIES ABQAIQ BI NO. 10-10303 |  |

| NO: | LOCATION | OBSERVATION | RECOMMENDATION | RESPONSIBLE ENTITY | DUE DATE | TYPE | STATUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | GOSP - 03 | No emergency eyewash station was available in the saltwater work area. | A portable, shaded eyewash station was installed in a clearly marked, accessible location. | Civil Supervisor | 30/08/2026 | Major | Closed |
| 23 | LAYDOWN | Scaffolding material was stored at the edge of an excavation, creating a surcharge load. | All materials were removed from the edge of the excavation to a designated storage area. | Scaffolding Supervisor | 03/09/2026 | Major | Closed |
| 24 | LAYDOWN | Wooden planks with protruding nails were found in the work area. | All wood with protruding nails was collected, and the nails were removed or bent over. | Rigger 3 | 03/09/2026 | Minor | Closed |

| _____________________________ Observed By: Safety officer Name: BILAL SAEED MUHAMM Date: 30/08/2026 *Note: Type – Minor & Major Status – Open & Closed | _____________________________ Status By: Site Engineer Name: AAFAQ AHMAD Date: 30/08/2026 | _____________________________ Reviewed By: Project Manager Name: AHMED GHALWASH Date: 05/09/2026 |
| --- | --- | --- |

EWI 107 Att.2 Rev.1 04 Sep. 2019